MATERIAL SAFETY DATA SHEET

U.S. Dept. of Labor

OSHA Hazard Communication Standard 29 CFR l9l0.1200

- Product Identification

Product Name: Mr. Hard Water Stain Remover 
Trade Name: Mr. Hard Water Stain Remover 
Common Names: Stain Remover

Identification: JFlint Products, Co. 68805 Perez Rd. F-42 Cathedral City, CA 92234

For Information, Call: (760) 770-3014
Emergency Number: (760) 770-3014
For Chemtrec Assistance, call: 800-424-9300 
For International Chemtrec Assistance, call: 703-527-3887

- Composition/Information on Ingredients

| Component | CAS No. | % | EINECS# |
| --- | --- | --- | --- |
| Soda | 497-19-8 | 30-50% | unlisted |
| Silica | 14464-46-1 | 50% | unlisted |

Hazard Symbols: None listed. 
Risk Phrases: None listed.

- Hazards Identification

EMERGENCY OVERVIEW
Appearance: Grey  May cause mechanical eye and skin irritation.

Target organs: None.

Potential Health Effects:

Eye: Dust may cause mechanical irritation.

Skin: Dust may cause mechanical irritation.

Ingestion: Expected to be a low ingestion hazard.

Inhalation: Low hazard for usual industrial handling.

Chronic: No information found.

- First Aid Measures

Eyes: Get medical aid. Do not allow victim to rub or keep eyes closed. Gently lift eyelids and flush continuously with water. 

Skin: Get medical aid. Flush skin with plenty of soap and water. 

Ingestion: If victim is conscious and alert, give 2-4 cupfuls of milk or water. Get medical aid if irritation or symptoms occur. 

Inhalation: Remove from exposure to fresh air immediately. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical aid if cough or other symptoms appear. 

Notes to Physician: Treat symptomatically.

- Fire Fighting Measures

General Information: As in any fire, wear a self-contained breathing apparatus in pressure-demand, msha/NIOSH (approved or equivalent), and full protective gear.

Extinguishing Media: Use water fog, dry chemical, Carbon Dioxide, or regular foam.

Autoignition temperature: Not available.

Flash Point: 82 deg c ( 179.60 Deg f)

Explosion Limits, Lower: Not available.

Explosion Limits, Upper: Not available.

NFPA Rating: not published.

- Accidental Release Measures

General Information: use proper personal protective equipment as indicated in 8.

Spills/leaks: Vacuum or sweep up material and place into a suitable disposal container.

- Handling and Storage

Handling: Use with adequate ventilation.
Storage: Store in a dry area.

- Exposure Controls/Personal Protection

Engineering Controls: Good general ventilation should be sufficient to control airborne levels.

OSHA vacated pels: Silica: No OSHA vacated pels are listed for this chemical.

Personal protective equipment

Eyes: Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 cfr 1910.133 Or European standard en166.

Skin: Protective garments not normally required.

Clothing: Protective garments not normally required.

Respirators: Follow the OSHA respirator regulations found in 29cfr 1910.134 Or European standard en 149. Always use a NIOSH or European standard en 149 approved respirator when necessary.